IMPACT OF MULTILAYER EXPLOSIVE CHARGE ON A RIGID WALL

A. A. Shtertser

Design and Technology Division, Lavrent’ev Institute of Hydrodynamics, 
Siberian Branch, Russian Academy of Sciences, ul. Tereshkovoi 29, 
Novosibirsk, 630090 Russia

e-mail: asterzer@mail.ru

The engineering formulas were derived to estimate the pressure and pulse (impact momentum) affecting a rigid wall at detonation of one- and multi-layer high explosive (HE) charges in 1D and 2D geometries. Rigid wall simulates any structural design which is subjected to HE impact. Explosive is characterized by density е, detonation velocity DH, and adiabatic exponent  of detonation products. Pressure at the Chapman–Jouguet plane is considered as the detonation pressure pH = eDH2/(+1), where γ = 2.8, 2.5, and 2.2 for RDX, amatol, and 1 : 1 amatol–ammonium nitrate mixture, respectively. In case of normally incident detonation wave (DW) on the wall, initial pressure p0 can be calculated using the formula:

p0/pH = [5γ + 1 + (17γ2 + 2γ + 1)1/2]/(4γ),

where γ is relevant to the HE layer contacting the wall. Actually, for all HE it can be assumed that p0/pH ≈ 2.4. When DW propagates from the wall towards to the open end of explosive charge (ignition at the wall), then

p0/pH = [(γ + 1)/(2γ)]2γ/(γ–1).

In this case, p0/pH ≈ 0.3. The pressure pulse transmitted to the wall does not depend on the direction of DW propagation and is calculated for uniform HE charge from the formula:

J = k(γ)ρeδeDH = k(γ)meDH,

where δe is the explosive charge thickness, me explosive charge mass per unit area, and

k(γ) = {3/[4(γ2 – 1)]}1/2.

For the multilayer charge consisting of a set of different HE layers, the total pulse can be accurately calculated using the formula:

J = ΣJi = Σk(γi)meiDHi.

In case of sliding detonation (planar geometry), the DW front is perpendicular to the wall, the initial pressure on the wall is equal to detonation pressure: p0 = pH. In the formula for pressure pulse,

k(γ) = [1/(γ + 1)]·{3/[2(γ – 1)]}1/2.

Fig. 1. Detonation in three-layer charge. Detonation velocity drops with decreasing layer number. Here α1, α2, α3 are inclination angles; 1 front of expanding detonation products; and 2 rigid wall.

In case of multilayer charge with different DW front angular inclinations, the total pressure pulse is given by the expression:

J = ΣJi = Σk(γi, αi)meiDHi,

where k(γ, α) = {[3/(4(γ2 – 1))]·[1 – (γ –1)(sin2α)/(γ + 1)]}1/2 (Fig. 1).

The suggested formulas are valid for numerous applications, including explosive welding [1, 2].

- Deribas, A.A., Fizika uprochneniya i svarki vzryvom (Physics of Explosive Welding and Strengthening), Novosibirsk: Nauka, 1980.

- Deribas, A.A. and Zakharenko, I.D., Surface phenomena at oblique collisions of metallic plates, Fiz. Goreniya Vzryva, 1974, vol. 10, no. 3, pp. 34–40.